UNIVERSITY OF NEWCASTLE UPON TYNE

SCHOOL OF CIVIL ENGINEERING & GEOSCIENCES

Summary Sheet - Weights, Concentrations, Equivalents

Weights

weight per volume 	(W/V) 		e.g.	mg/l

weight per weight	(W/W)		e.g.	mg/kg = ppm	(parts per million)

g/kg = ppb (parts per billion)

if solvent has density of 1  e.g. water then ppm = mg/l and ppb = g/l

molecular weight = sum of atomic weights

e.g. MWt of NH3 = 14 + 1 + 1 + 1 = 17g/mole

equivalent weight = molecular weight divided by z		EW=MW/z

(z is 1,2,3 etc depending on the reactive species)	units are g/equivalent (or g/eq)

the number of equivalents in a given weight of substance is given by:

weight of material (in grams) / equivalent weight

Weight as Amount of Substance

Q. how to add  NH3 values in different units 100 mg/l and 5 meq/l ?

all expressions of weight must be in the same units before they can be added

100 mg NH3/l is the same as  100 x 14/17 = 82.3 mg NH3-N/l

5 milliequivalents of NH3 per litre (5 meq NH3/l) 			= 5 x 1/1000 eq NH3/l

as 1 equivalent of NH3 weighs 17g. (i.e.17gleq)		= 5 x 1/1000 x 17 g NH3 /l

as 17g NH3 contains 14 g N			= 5 x 1/1000 x 17 x 14/17 g NH3-N /l

as there are1000mg in 1g 			= 5 x 1/1000 x 17 x 14/17 x 1000 mg NH3-N /l

= 70 mg NH3-N /l

Concentration

molar = moles per litre, units are M

normal = equivalents per litre, units are N

to convert M to N simply use the factor z 		e.g.	M x z = N				remember 	 1N is one equivalent weight in 1 litre

 a solution's normality N is always  its molarity M

Titrations

Cs.Vs = Ct.Vt	  (where s indicates sample, and t indicates titrant)

i.e. concentration times volume of titrant = concentration times volume of sample

but concentrations must be in units of Normality N  (i.e. eq/l)

 rearrange and multiply by EW to convert concentration from units of N to g/litre (or mg/l)

Cs  = Ct.Vt .EW / Vs   (Cs in g/l)

Cs  = Ct.Vt .EW.1000 / Vs   (Cs in mg/l)